==========================================================================

README file for EPSON Stylus(R) High Quality ESC/P 2 printer driver

for Microsoft (R) Windows (R) 2000

==========================================================================

Thank you for purchasing an EPSON printer.  This document contains notes regarding your

new printer.

This README file has the following sections:

[Contents]

1. Introduction

2. Important legal information

3. Driver Installation Notes

4. Q & A

5. Application Notes

6. Known printer driver problems

--------------------------------------------------------------------------------------------------------------------------

1. Introduction

==========================================================================

Before calling technical support, be sure to read this README file, which includes

solutions to printer driver and application-specific problems.

2. Important legal information

==========================================================================

Software called the EPSON Stylus High Quality ESC/P 2 driver for Windows 2000

(the "Software") is provided "AS IS."  NO IMPLIED WARRANTIES OF

MERCHANT ABILITY AND/OR FITNESS FOR A PARTICULAR PURPOSE ARE

GIVEN.

The Software may be copied and distributed without charge only to users of EPSON

brand printers. All files on the disk must be copied without modification, including all

proprietary marks and notices. All components of the Software must be distributed

together. The Software may not be distributed for profit.

Copyright (c) 1999 SEIKO EPSON Corporation

3. Driver Installation Notes

==========================================================================

First time installation:

Insert the EPSON printer software CD in your CD-ROM drive. Click the Start button

and select RUN. Type in the appropriate driver letter that identifies your CD ROM drive.

Click Browse and select the SETUP.EXE file and press Enter.  Select the EPSON printer

driver then click OK.

Removing driver:

When you're reinstalling the printer driver or installing a printer driver for a second

port, make sure to do the following:

* Check that all print jobs have finished printing before you begin installation.

* Close all applications

* Run the "EPSON Printer Utility Uninstall" program by the following steps:

- Click on "Start"

- Click on "Settings"

- Select "Control Panel"

- Double click the "Add/Remove Programs" icon

- Select the "EPSON Printer Software" from the list and click

"Change/Remove"

4. Q & A

==========================================================================

Q: The error message "Insufficient memory to print. Please consult the 'ReadMe' file

for more information." is displayed, printing cannot be carried out. Why?

A: There is insufficient memory for printing. To resolve this problem try the following:

* Expand the installed Memory

* Expand the size of the Virtual Memory

For details on this method please consult the Windows 2000 Help page

titled "To change the size of the virtual memory paging file"

Q: The error message "Insufficient disk space to print. Please consult the 'ReadMe' file

for more information." is displayed, printing cannot be carried out. Why?

A: There is insufficient free hard disk space to carry out the print job.

To resolve this problem try the following:

* Expand the amount of free space on your Hard Disk

Q: The error message "A Fatal Error has occurred. Unable to print. Please consult the

'ReadMe' file for more information." is displayed, printing cannot be carried out.

Why?

A: The printer driver is not installed correctly. As per the instructions listed in Section 3

(Driver Installation Notes) of this readme, please re-install the printer driver.

Q: Cannot print. There is no error message displayed.

A: If you  print a document with image data, like Tiff file, after specifying the resolution

of either 1440dpi or 720dpi where "Finest Detail" has been selected in the driver

settings, some Drawing software creates a large quantity of interim print data.

To print with software like this, a large quantity of both Virtual Memory and free HD

space is necessary.

Without enough Virtual Memory and free HD space, the software can't allocate the

required Memory and may possibly lose the print data during spooling and/or greatly

slow down the print job.

In such a case, to avoid the problem:

* Expand the installed Memory

* Expand the size of the Virtual Memory

For details on this method please consult the Windows 2000 Help page

titled "To change the size of the virtual memory paging file"

* Expand the amount of free space on your Hard Disk

Q: The output on my paper never appears to the settings I use in my application software.

A: Settings that are changed in the printer driver override the settings made in your

application software. For example, when the Print Quality is set to Fine-720 dpi with

Plain paper selected as the Media Type, your documents are printed at Fine-720 dpi

even though the resolution in your application software is set to another resolution.

Q: Color Gradations (Gradual shift in color) are not printed correctly. Can this be

corrected?

A: This problem occurs as result of a difference between the Application color

handling method and the Driver Automatic Color Correction method.

Try following the steps below to correct the problem.

1) From the application's File menu, select Print.

2) Click Properties.

3) Select Custom from the Main menu.

4) Click Advanced.

5) Select Photo-realistic or Vivid from  Color Adjustment - Mode selection.

5. Application notes

==========================================================================

* Lotus WordPro 96

1. Very rarely, an imported bitmap image is not printed correctly.

2. The frame line is printed incorrectly. (All or some parts of the frame line exceed the document

dimensions)

* Works 4.0

If the "Cancel" button is clicked while printing, a "memory low" message is sometimes

displayed.

* Excel 97

Color of pattern occasionally looks different from print preview.

* WordPad

When printing a document just after system start-up the print position is sometimes incorrect

(skipped page/skipped column).  This problem has been noted only on the 1st print job.

* MS-Graph(MS-Word)

The name of a graph in a document is sometimes printed incorrectly.  The position of the graph

name is different from the screen.

* Corel Draw 6.0

Very rarely: An imported photographic image is not printed correctly.

* MS-Word 7.0

Sometimes the frame line (dotted line) color is different from the screen.

* Lotus1-2-3  5.0

Sometimes the line feed position is different from the preview.

* PowerPoint 7.0

The background color of a document is not printed correctly when printing using gray.

* Microsoft Word 6.0, Microsoft Works 4.0, CorelDRAW 7.0, PowerPoint 7.0

Changing the copy count in the "copies" field on the paper dialog box does not change the

# of copies printed.

6. Known printer driver problems

==========================================================================

There are no known bugs within the printer driver itself.

--------------------------------------------------------------------------------------------------------------------------

For the latest information regarding your EPSON printers, visit the EPSON Web

Site at http://www.epson.com. Go to the EPSON Connection page and select FAQs,

Inkjet Printers. Or you can access one of the following EPSON Electronic

Support Services:

Fax-On-Demand:

Dial (800) 442-2110 or (310) 782-4214 select option 1, then 1 again

(to order a document).

Service:

File Area: Inkjet Printers

CompuServe:

Forum: EPSON

Section: Inkjet Printers

--------------------------------------------------------------------------------------------------------------------------

Microsoft is a registered trademark and Windows is a trademark of Microsoft

Corporation in the United States and other countries.  Product and Company

names listed in this document are registered trademarks of said Company.